Dokumen API Radar X-Band New

1. Document Overview
This document specifies the communication protocol for the RDXXB series radar and display control system.

2. Communication Agreement
The communication between the radar and display control system uses the UDP network protocol.
The display control system’s IP address is: 192.168.1.2, with the local port number 8000;
The radar’s IP address is: 192.168.1.3, with the local port number 7000.
For data content exceeding one byte, it is processed in little-endian order, meaning the high byte comes after the low byte.
The display control system broadcasts commands to 255.255.255.255:11111, and the processing machine replies with the network configuration response frame to the corresponding IP and port.

3. Data Protocol

3.1 Frame Format
All communication data frames follow the format outlined below.

| Logo | Byte Count | Data Types | Data Content |
| --- | --- | --- | --- |
| Frame Header | 2 | unsigned Short | 0x55AA |
| Identifier | 2 | unsigned short | 0xFF01Tracking target information frame 0xFF02: Search target information frame 0xFF03Status information frame 0xFF04Parameter update frame 0xFF05Scan mode switching command frame 0xFF30Reserved (This content is prohibited from being sent) 0xFF31Reserved (This content is prohibited from being sent) 0xFF32Reserved (This content is prohibited from being sent) 0xFF33Network configuration command/response frame 0xFF34Reserved (This content is prohibited from being sent) |
| Frame Count | 4 | unsigned int | 0x00000000~0xffffffffThe loop is synchronized by the receiver. |
| Frame Content Length | 4 | unsigned int | The size of the frame content, in bytes. |
| Frame Content | n |  |  |
| Protocol Identifier | 2 | unsigned char | 1:RD01 2:RD02 3:RD03 … 8:RD08 Different radar identifier protocols do not guarantee compatibility. If the software receives a protocol with an invalid identifier... Discussion, simply ignore. |
| Protocol Major Version Number | 1 | unsigned char | Main version number:1,2,3… If significant changes occur, the major version number will be upgraded; a higher major version number indicates a more sophisticated protocol. New software is compatible with older protocols, but older software is not guaranteed to be compatible with the new protocol (forward compatibility). (Incompatible with backwards) default value:2 |
| Protocol Sub-version Number | 1 | unsigned char | Sub-version number:0,1,2… If general changes occur, upgrade the sub-version number. If the major version number is the same, upgrade the sub-version number. A larger number indicates a newer protocol, ensuring that new software is compatible with older versions of the protocol, and older software is compatible with theProtocol (forward and backward compatible) default value:13 |
| Checksum | 1 | unsigned char | Sum all data except the frame header and take the least significant byte. (uint8_tAccumulation) |
| total | 16+n |  |  |

3.2Frame content

3.2.1Mode command frame

The mode command frame contains three operating modes: standby, search, and track. This frame is sent to the radar by the display and control system.

The radar defaults to standby mode when powered on.

The radar does not respond to repeated commands.

- Standby/Search Mode

surface Standby/Search Frame

| Logo | byte count | Data Types | Definition |
| --- | --- | --- | --- |
| mode type | 1 | unsigned char | 0x00Standby mode 0x11Search mode 0x33:Leveling mode (Level the machine before operation, and use it in machine scan or standby mode. Set the servo mode to loop scan mode in the issued command. Do not perform other operations during the leveling process. When a signal is received...)3.2.2Status information The mode is0x00or0x33,(Leveling complete) |
| spare | 4 | char | for internal use only. 0 |
| Frequency Selection | 1 | unsigned char | Value range: [0, 20] See appendix for details.1Frequency point correspondence table default value:10 |
| Silent Zone1 Starting Angle | 2 | unsigned short | 0°~360°precision0.1°,invalid7FFFH Range of values [0,3600] Corresponding status information frame:Radar front end1Azimuth Angle like180.5°, Send value1805 |
| Silent Zone1 End Corner | 2 | unsigned short | 0°~360°precision0.1°,invalid7FFFH Range of values [0,3600] Corresponding status information frame:Radar front end1Azimuth Angle |
| Silent Zone 2 Starting Angle | 2 | unsigned short | 0°~360°precision0.1°,invalid7FFFH default value:0x7FFF |
| Silent Zone 2 End Angle | 2 | unsigned short | 0°~360°precision0.1°,invalid7FFFH default value:0x7FFF |
| Distance compensation | 2 | Short | accuracy0.1Default fill0 like2.3Meters, sent value:twenty three |
| Azimuth compensation | 2 | Short | accuracy0.1Default fill0 like1.2°, Send value:12 |
| Pitch Compensation | 2 | Short | accuracy0.1Default fill0 like1.1°, Send value:11 |
| Scan Cycle | 2 | Short | Both standby and search modes are effective, with high accuracy.0.1Default fill3.2s,default value:32 |
| Servo Mode | 1 | unsigned char | Both standby and search modes are effective. 0x11Represents pointing mode 0x33Represents circular scanning mode 0x77Represents pulling out the locking pin 0x88Represents inserting a pin 0xFFRepresents idle mode Other instructions will be executed according to the idle instruction. Note: Standby command0xFF;Search/leveling commands:0x33 Pointing instruction:0x11 |
| Servo Pointing Angle | 2 | unsigned Short | Both standby and search modes are effective. 0~3600,represent0~360.0degree, accuracy0.1The range of values like180.5°, Send value1805 |
| System Control Flags | 1 | unsigned char | Only valid in standby mode 0x00This means that the operation corresponding to the system control code will not be executed. 0x01This represents the operation corresponding to the system control code. Others, according to0x00implement |
| System Control Code | 1 | unsigned char | Only valid in standby mode 0x01The system control operation is to shut down the radar. Other, no action. |
| Manual Pitch Enable | 1 | char | 0x00 :Disable0x011:Enable (Values are:)0x011At this time, the angle obtained by leveling will no longer be used, and the set array will be used instead. (Surface mounting pitch angle) default value:0 |
| spare | 8 | char |  |
| longitude | 8 | double | reserve6Decimal places (setup point)GPSinformation) |
| latitude | 8 | double | reserve6Decimal places (setup point)GPSinformation) |
| high | 8 | double | reserve6Decimal places (setup point)GPSinformation) |
| range | 1 | char | 0x11:7.5km;0x22:15km default value:0x11 |
| spare | 256 | char |  |
| 3 Km Inner Height Lower Limit | 2 | short | accuracy1mUsed for rejection3kmWithin (inclusive)3kmTargets at altitudes lower than this height default value:0 |
| 3 Km External Height Lower Limit | 2 | short | accuracy1mUsed for rejection3kmTargets at altitudes lower than this height default value:100 |
| Height Limit | 2 | short | accuracy1mRemove targets with a height not less than this value. default value:600 |
| Speed Lower Limit | 2 | unsigned short | accuracy0.1m/sEliminate targets with absolute speeds lower than this value. default value:0 |
| Speed Limit | 2 | unsigned short | accuracy0.1m/sEliminate targets with absolute speeds greater than this value. like12.3 m/sSend value:123 default value:600 |
| spare | 2 | unsigned char |  |
| Target Reporting Distance Limit | 2 | unsigned short | accuracy1mRemove targets that are more than this distance. default value:7500 |
| Target Reporting Distance to Lower Limit | 2 | unsigned short | accuracy1mRemove targets that are less than this distance. default value:200 |
| spare | 30 | unsigned char |  |
| Roll Angle Installation on Array | 4 | float | Accurate to the decimal point2Position, corresponding to the built-in inclinometerXThe axis, viewed along the radial direction, is lower on the left and lower on the righHigh is positive, and left higher than right is negative. (3.2.2Status information:Roll angle of the formation) default value:0.0 |
| Elevation Angle of Array Installation | 4 | float | Accurate to the decimal point2Position, corresponding to the built-in inclinometerYThe axis, viewed along the radial direction, is lower in the front and lower in the bacHigh is positive, high in the front and low in the back is negative. (3.2.2Status information:Pitch angle of the array) default value:0.0 |
| North Corner of the School | 4 | float | Accurate to the decimal point2Bit Value range: [0,360] default value:0.0 InstallGPSAnd if the heading validity mark is valid, the calculation shall be performed with reference to Appendix II. |
| Autonomous identification enablement | 1 | char | 0: Not enable1Enable (If not enabled, target identification will only be performed by tracking the flight path; otherwise...) Search tracks are also identified, and the search period will be longer. default value:1 |
| Autonomously Identify the Maximum Number of Targets | 1 | char | scope:1~2 default value:2 |
| reserve | 30 | char |  |
| total |  |  |  |

- Tracking Mode

| Label | Byte Count | Data Types | Definition |
| --- | --- | --- | --- |
| Mode Type | 1 | unsigned char | 0x22: Tracking mode |
| Tracking Type | 1 | unsigned char | 0x11: Enter tracking 0x22: Cancel tracking |
| Number of Targets Tracked by Radar | 2 | unsigned short | Fixed as 1 |
| Spare | 128 | char |  |
| Target Backup | 16 | char |  |
| Target Number | 2 | unsigned short |  |
| Target Distance | 4 | float |  |
| Target Location | 4 | float |  |
| Target Pitch | 4 | float |  |
| Target Height | 4 | float |  |
| Target Radial Velocity | 4 | float |  |

3.2.2Status information frame

Users can selectively display content as needed.

The period for sending this status frame is...200msLeft and right (not a strict time interval).

surface4Status information frame

| Label | Byte Count |  | Data Types |  | Definition |  |
| --- | --- | --- | --- | --- | --- | --- |
| Work Mode | 1 |  | unsigned char |  | 0x00: Standby 0x11: Search 0x22: Track 0x33: Leveling |  |
| Command Execution Status | 1 |  | unsigned char |  | 0x00: No new commands received 0x11: Executing 0x22: Execution successful 0x33: Execution failed, Other, invalid state |  |
| Fault Type | 1 |  | unsigned char |  | 0x00: No faults 0x01: Command parsing failed 0x02: Command could not be executed, Other, custom faults |  |
| Radar Model | 1 |  | unsigned char |  | 0x01–0x10: Corresponding to RD01–RD10 |  |
| Number of Radar Front-End IDs | 1 |  | unsigned char |  | Fixed value: 0x01 |  |
| Radar Front-End Network Status | 1 |  | unsigned char |  | 0: Not connected 1: Connected |  |
| Radar Front-End Scan Cycle | 4 |  | unsigned int |  |  |  |
| Radar Front-End Azimuth Angle | 4 |  | float |  | [0, 360] |  |
| Radar Front-End Pitch Angle | 4 |  | float |  | [0, 360] |  |
| Radar Front-End Operating Frequency | 2 |  | unsigned short |  | 0–20, corresponds to 2GHz, 20 corresponds to 9.8GHz |  |
| Spare | 16 |  | char |  |  |  |
| Radar Front-End Silent Zone1 Rise | 2 |  | short |  | 0°–360° precision 0.1°, invalid 0x7FFF (Valid scope: [0,360]) Such as received value 1805, expressed: 180.5 |  |
| Radar Front-End Silent Zone1 Knot | 2 |  | short |  | 0°–360° precision 0.1°, invalid 0x7FFF (Valid scope: [0,360]) Such as received value 1805, expressed: 180.5 |  |
| Radar Front-End Silent Zone2 Rise | 2 |  | short |  | 0°–360° precision 0.1°, invalid 0x7FFF (Valid scope: [0,360]) Such as received value 1805, expressed: 180.5 |  |
| Radar Front-End Silent Zone2 Knot | 2 |  | short |  | 0°–360° precision 0.1°, invalid 0x7FFF (Valid scope: [0,360]) Such as received value 1805, expressed: 180.5 |  |
| Fan Status | 1 |  | unsigned char |  | Bit[1:0]: Indicates fan1 state, Bit[3:2]: Indicates fan2 state, Bit[5:4]: Indicates fan3 state. 0 indicates normal, 1 indicates an anomaly. |  |
| Frequency Synthesis State | 1 |  | unsigned char |  | Bit0-expressLO1state, Bit1-expressLO2state, 0: Indicates an anomaly, 1: Normal |  |
| Sub-array1 Temperature | 2 |  | short |  | accuracy1 Celsius |  |
| Sub-array2 Temperature | 2 |  | short |  | accuracy1 Celsius |  |
| Sub-array3 Temperature | 2 |  | short |  | accuracy1 Celsius |  |
| Sub-array4 Temperature | 2 |  | short |  | accuracy1 Celsius |  |
| Sub-array5 Temperature | 2 |  | short |  | accuracy1 Celsius |  |
| Sub-array6 Temperature | 2 |  | short |  | accuracy1 Celsius |  |
| Sub-array7 Temperature | 2 |  | short |  | accuracy0.1A |  |
| Sub-array8 Temperature | 2 |  | short |  | accuracy0.1A |  |
| Information Status | 1 |  | unsigned char |  | Bit0-Representation and ChannelADOver-range0-This indicates that the measurement range has not been exceeded.1-This indicates that the range has been exceeded. Bit1-Indicates pitch difference channelADOver-range0-This indicates that the measurement range has not been exceeded.1-This indicates that the range has been exceeded. Bit2-Indicates azimuth difference channelADOver-range0 -This indicates that the measurement range has not been exceeded.1-This indicates that the range has been exceeded. Bit3-Indicates the backup channelADOver-range0 -This indicates that the measurement range has not been exceeded.1-This indicates that the range ha Bit4-expressQDRInitialization complete.0 -Indicates incomplete.1-This indicates completion. Bit5- expressDDR-0Initialization complete.0 -Indicates incomplete.1-This indicates completion. Bit6-express DDR-1Initialization complete.0 -Indicates incomplete.1-This indicates completion. Bit7-expressPCIeLink status,0 -Indicates incomplete.1-This indicates completion. |  |
| Signal Voltage1 | 2 |  | unsigned short |  | accuracy1V |  |
| Signal Voltage2 | 2 |  | unsigned short |  | accuracy1V |  |
| Signal Voltage3 | 2 |  | unsigned short |  | accuracy1V |  |
| Temperature at Several Locations | 4 |  | float |  | accuracy1 Celsius |  |
| Pitch Frame Angle | 4 |  | float |  | [0,360] |  |
| Pitch Adjustment Level | 4 |  | float |  | [0,360] |  |
| Spare | 348 |  | char |  | String |  |
| Servo Mode | 1 |  | char |  | 0x11: Pointing mode 0x12: Circular scanning mode 0x27: Pulling out the locking pin 0x88: Inserting a pin |  |
| Latitude of the Erection Point | 8 |  | double |  | Latitude of the erection point |  |
| Longitude of the Erection Point | 8 |  | double |  | Longitude of the erection point |  |
| Elevation of the Erection Point | 4 |  | float |  | Elevation of the installation point |  |
| Heading Angle | 4 |  | float |  | The geographical azimuth (heading angle) as defined by the radar array normal. |  |
| Valid Heading Mark | 1 |  | unsigned char |  | 0: Invalid, 1: Efficient |  |
| Number of Satellites | 1 |  | unsigned char |  | The number of satellites detected |  |
| North Correction | 2 |  | unsigned short |  | The north correction angle, calculated autonomously by the radar, can be used directly by the user in standby mode. Range: 0–360° |  |
| Roll Angle of the Formation | 4 |  | float |  | Corresponding built-in inclinometer |  |
| Pitch Angle of the Array | 4 |  | float |  | Corresponding built-in inclinometer |  |
| Q/Y/Z | 4 |  | float |  |  |  |
| HH | 2 |  | short |  | Hour |  |
| MM (Month) |  | 2 |  | short |  |
| SS (Seconds) |  | 2 |  | short |  | Milliseconds (range: 90–99) represent 0.90ms precision (10ms) |
| M5 (Milliseconds) |  | 2 |  | short |  | Milliseconds (range: 90–99) represent 0.90ms precision (10ms) |
| YEAR |  | 2 |  | short |  | Year |
| MONTH |  | 2 |  | short |  | Month |
| DAY |  | 2 |  | short |  | Day |
| Servo Status |  | 1 |  | unsigned char |  | Bit0 indicates normal, Bit1: Anomaly status |
| Scanning Mode |  | 1 |  | unsigned char |  | Bit0 indicates sweep mode, Bit1: Phase sweep, Bit2: Backup sweep |
| Radar IP Address |  | 16 |  | unsigned char |  | IP Address in ASCII format |
| Radar Port Number |  | 2 |  | unsigned short |  | Radar port number |
| Client IP Address |  | 16 |  | unsigned char |  | Client’s IP address in ASCII format |
| Client Port Number |  | 2 |  | unsigned short |  | Client’s port number |
| Reserve |  | 4 |  | char |  |  |

3.2.3Search target information frame

Each radar pulse group sends target information to the display and control system.

This frame is of variable length, its length depending on the number of targets. Even when no target is found in the current pulse group, a target search header is still sent.

At this time, there is no target content information.

surface5Search target information header (head)

| Label | Byte Count | Data Type | Definition |
| --- | --- | --- | --- |
| Search Location | 4 | float |  |
| Search Overview | 4 | float |  |
| Search Cycle | 4 | unsigned int |  |
| Current Page Number | 4 | unsigned int |  |
| Target Number | 2 | unsigned short | Report target numberN |
| Spare | 2 | unsigned char | 0x0000 |
| Total | 20 |  |  |

Table: SurfaceSearch Target Information Main Content (Body)

| Label | Byte Count | Data Type | Definition |
| --- | --- | --- | --- |
| TargetTrack Deletion Marker | 2 | unsigned short | 0x00: Do not delete flight tracks 0x01: Track deletion |
| TargetTrack Number | 2 | unsigned short | 0–Maximum number (10000) (Tentative) |
| TargetLoss Count | 2 | unsigned short |  |
| Target Reason for Loss | 2 | unsigned short |  |
| Target Update Time | 4 | unsigned int | Unit: CPI count |
| Target Energy | 4 | float | 32-bit binary floating point number |
| Target Credit Ratio | 4 | float | 32-bit binary floating point number |
| Target Speed | 4 | float | 32-bit binary floating point number |
| Target Distance | 4 | float | 32-bit binary floating point number |
| Target1 Position | 4 | float | 32-bit binary floating point number |
| Target1 Looking Up and Down | 4 | float | 32-bit binary floating point number |
| Target1 High | 4 | float | 32-bit binary floating point number |
| Target1 Track Type | 1 | unsigned char | 0x00: Unknown target 0x11: People 0x12: Car 0x20: Drones 0x24: Bird 0x31: Vessel 0x40: Other objectives 0x41: False target |
| Target1 Attributes (Search) | 1 | unsigned char |  |
| Target1 Radar ID | 1 | unsigned char |  |
| Spare1 | 3 | unsigned char |  |
| Target1 Track Points | 2 | unsigned short |  |
| Target Longitude | 8 | double | Reserved decimal places |
| Target Latitude | 8 | double | Reserved decimal places |
| Target1 High | 8 | double | Reserved decimal places |
| Target1 Vx | 4 | float |  |
| Target1 Vy | 4 | float |  |
| Target1 Vz | 4 | float |  |
| Target1 Spare | 22 | unsigned char |  |
| Target1 Envelope Points | 1 | unsigned char |  |
| Spare2 | 13 | unsigned char |  |
| Target1 UTC Time Years | 2 | short | Year |
| Target1 UTC Time Month | 1 | char | Moon |
| Target1 UTC Time Day | 1 | char | Day |
| Target1 UTC Time Hour | 1 | char | Hour |
| Target1 UTC Time Division | 1 | char | Point (second) |
| Target1 UTC Time in Seconds | 1 | char | Scope: 0–99, represents 0–990ms precision 10ms |
| Target1 UTC Time in Milliseconds | 1 | char |  |
| Target1 Spare2 | 19 | unsigned char |  |
| Target1 Confidence | 1 | unsigned char | Accuracy: 0.01 |
| TargetN Track Deletion Marker | 2 | unsigned short | 0x00: Do not delete flight tracks 0x01: Track deletion |
| TargetN Track Number | 2 | unsigned short | 0–Maximum number (10000) (Tentative) |
| TargetLoss Count | 2 | unsigned short |  |
| Target Reason for Loss | 2 | unsigned short |  |
| Target Update Time | 4 | unsigned int | Unit: CPI count |
| Target Energy | 4 | float | 32-bit binary floating point number |
| Target Credit Ratio | 4 | float | 32-bit binary floating point number |
| Target Speed | 4 | float | 32-bit binary floating point number |
| Target Distance | 4 | float | 32-bit binary floating point number |
| Target Position | 4 | float | 32-bit binary floating point number |
| Target Looking up and Down | 4 | float | 32-bit binary floating point number |
| Target High | 4 | float | 32-bit binary floating point number |
| Target Track Type | 1 | unsigned char | 0x00: Unknown target 0x11: People 0x12: Car 0x20: Drones 0x24: Bird 0x31: Vessel 0x40: Other objectives |
| Target Attributes (Search) | 1 | unsigned char |  |
| Target Radar ID | 1 | unsigned char |  |
| Spare1 | 3 | unsigned char |  |
| Target Track Points | 2 | unsigned short |  |
| Target Longitude | 8 | double | Reserved decimal places |
| Target Latitude | 8 | double | Reserved decimal places |
| Target High | 8 | double | Reserved decimal places |
| Target NVx | 4 | float |  |
| Target NVy | 4 | float |  |
| Target NVz | 4 | float |  |
| Target Spare1 | 22 | unsigned char |  |
| Target Envelope Points | 1 | unsigned char |  |
| Spare2 | 13 | unsigned char |  |
| Target UTC Time Years | 2 | short | Year |
| Target UTC Time Month | 1 | char | Moon |
| Target UTC Time Day | 1 | char | Day |
| Target UTC Time | 1 | char | Hour |
| Target UTC Time Division | 1 | char | Point (second) |
| Target UTC Time in Seconds | 1 | char | Scope: 0–99, represents 0–990ms precision 10ms |
| Target UTC Time in Milliseconds | 1 | char |  |
| Target Spare2 | 19 | unsigned char |  |
| Target Confidence | 1 | unsigned char | Accuracy: 0.01 |

3.2.4Tracking target information frame

The radar sends target tracking information to the display and control system. Even when there is no target in the current pulse group, it still sends target head tracking information; at this time, no...

There is a target content information section.

surface7Tracking target information header (head)

| Label | Byte Count | Data Type | Definition |
| --- | --- | --- | --- |
| Search Location | 4 | float |  |
| Search Overview | 4 | float |  |
| Search Cycle | 4 | unsigned int |  |
| Current Page Group Number | 4 | unsigned int |  |
| Target Number | 2 | unsigned short | Report the target number (mechanical scanning mode is fixed as follows) |
| Spare | 2 | unsigned char | 0x0000 |
| Total | 20 |  |  |

Table: SurfaceTracking Target Information Main Content (Body)

| Label | Byte Count | Data Type | Definition |
| --- | --- | --- | --- |
| Track Deletion Marker | 2 | unsigned short | 0x00: Do not delete flight tracks 0x01: Track deletion |
| Track Number | 2 | unsigned short | 0–Maximum number (10000) (Tentative) |
| Loss Count | 2 | unsigned short |  |
| Reason for Loss | 2 | unsigned short |  |
| Update Time | 4 | unsigned int | Unit: CPI count |
| Target Energy | 4 | float | 32-bit binary floating point number |
| Target Signal to Noise Ratio | 4 | float | 32-bit binary floating point number |
| Target Speed | 4 | float | 32-bit binary floating point number |
| Target Distance | 4 | float | 32-bit binary floating point number |
| Target Location | 4 | float | 32-bit binary floating point number |
| Target Pitch | 4 | float | 32-bit binary floating point number |
| Target Height | 4 | float | 32-bit binary floating point number |
| Target Track Type | 1 | unsigned char | 0x00: Unknown target 0x11: People 0x12: Car 0x20: Drones 0x24: Bird 0x31: Vessel 0x40: Other objectives |
| Target Attributes (Search) | 1 | unsigned char |  |
| Target Radar ID | 1 | unsigned char |  |
| Spare1 | 3 | unsigned char |  |
| Target Track Points | 2 | unsigned short |  |
| Target Longitude | 8 | double | Reserved decimal places |
| Target Latitude | 8 | double | Reserved decimal places |
| Target Height | 8 | double | Reserved decimal places |
| Target1 Vx | 4 | float |  |
| Target1 Vy | 4 | float |  |
| Target1 Vz | 4 | float |  |
| Target Backup1 | 22 | unsigned char |  |
| Target Envelope Points | 1 | unsigned char |  |
| Spare2 | 13 | unsigned char |  |
| Target UTC Time Years | 2 | short | Year |
| Target UTC Time Month | 1 | char | Moon |
| Target UTC Time Day | 1 | char | Day |
| Target UTC Time | 1 | char | Hour |
| Target UTC Time Division | 1 | char | Point (second) |
| Target UTC Time in Seconds | 1 | char | Scope: 0–99, represents 0–990ms precision 10ms |
| Target UTC Time in Milliseconds | 1 | char |  |
| Target Spare2 | 19 | unsigned char |  |
| Target Confidence | 1 | unsigned char | Accuracy: 0.01 |
| Target UTC Time in Milliseconds | 1 | char | Scope: 0–99, represents 0–990ms precision 10ms |
| Target Backup2 | 20 | unsigned char |  |
| Total | N*148 |  |  |

3.2.5Parameter update frame

When the radar is in search mode, the user can update some data processing thresholds. The radar will update the track based on the latest settings.

Information. This frame can be sent repeatedly.

surface9Parameter update frame

| Label | Byte Count | Data Types | Definition |
| --- | --- | --- | --- |
| Spare | 317 | char |  |
| 3kmInner Height Lower Limit | 2 | short | accuracy1m used for rejection 3km within (inclusive) Targets at altitudes lower than this height |
| 3kmExternal Height Lower Limit | 2 | short | accuracy1m used for rejection 3km Targets at altitudes lower than this height |
| Height Limit | 2 | short | accuracy1m removes targets with height not less than this value. |
| Speed Limit | 2 | unsigned short | accuracy1m limits targets with absolute speed lower than this value. |
| Spare | 2 | unsigned char |  |
| Target Reporting Distance Limit | 2 | unsigned short | accuracy1m remove targets that are more than this distance. |
| Target Reporting Distance Lower Limit | 2 | unsigned short | accuracy1m remove targets that are less than this distance. |
| Reserve | 74 | char |  |
| Total | 407 |  |  |

3.2.6Scan mode switching command frame

The scan mode switching command is sent from the display control to the data processing software. After receiving the command, the data processing software will safely exit the current

mode and enter the scan mode specified by the command. If a duplicate command is received, it will not respond.

The data processing software feeds back the current scanning method and status information to the display and control system in real time. The display and control system then displays and makes decisions based on the data.

The servo can only be controlled in mechanical scanning mode. Therefore, before switching to phase scanning mode, the display and control system must point the servo to the user's

expected pointing angle.

In phase scan mode, the current angle of the servo cannot be obtained by default.GPSAnd inclinometer data.

surface10Scan mode switching command frame

| Label | Byte Count | Data Types | Definition |
| --- | --- | --- | --- |
| Scanning Mode | 1 | unsigned char | 0x01: Enter mechanical scanning mode 0x02: Enter phase scan mode 0x03: Enter mechanical phase scan mode (backup) |
| Spare | 15 | unsigned char | Other values will be treated as mechanical scanning modes. |

Appendix 1:

| Serial Number | Frequency Code | X Frequency (GHz) | Ku Frequency (GHz) |
| --- | --- | --- | --- |
| 1 | 0 | 9.2 | 15.70 |
| 2 | 1 | 9.23 | 15.75 |
| 3 | 2 | 9.26 | 15.80 |
| 4 | 3 | 9.29 | 15.85 |
| 5 | 4 | 9.32 | 15.90 |
| 6 | 5 | 9.35 | 15.95 |
| 7 | 6 | 9.38 | 16.00 |
| 8 | 7 | 9.41 | 16.05 |
| 9 | 8 | 9.44 | 16.10 |
| 10 | 9 | 9.47 | 16.15 |
| 11 | 10 | 9.5 | 16.20 |
| 12 | 11 | 9.53 | 16.25 |
| 13 | 12 | 9.56 | 16.30 |
| 14 | 13 | 9.59 | 16.35 |
| 15 | 14 | 9.62 | 16.40 |
| 16 | 15 | 9.65 | 16.45 |
| 17 | 16 | 9.68 | 16.50 |
| 18 | 17 | 9.71 | 16.55 |
| 19 | 18 | 9.74 | 16.60 |
| 20 | 19 | 9.77 | 16.65 |
| 21 | 20 | 9.8 | 16.70 |

Appendix II:

In mechanical scanning standby mode:

Use the current heading angle and the previous30Compare with the mean of the second heading angle; if it is less than...0.01At that time, substitute the current heading angle into the following function to calculate, and...

Calculate the heading angle.

float north_angle(float servo_angle, float course_angle) {

float north = 0.0; float

angle = 0.0; angle =

course_angle - 90;

while(angle>360)

angle = 360;

while(angle<0)

angle += 360;

north = servo_angle - angle;

while(north>360)

north-=360;

while (north < 0)

north+=360;

return north;

}

servo_angle:3.2.2Status information frame radar front end1Azimuth Angle

course_angle:3.2.2State information frame heading angle